Hardware Setup Guide

Humanoid Robot — Raspberry Pi 4 + PCA9685

## Overview

This guide covers the electrical wiring between the Raspberry Pi 4 and the PCA9685 16-channel PWM servo driver, including servo motor power supply connections.

You will need:

- Raspberry Pi 4 (any RAM variant)

- Adafruit PCA9685 16-channel 12-bit PWM servo driver

- 5V–6V regulated power supply (for servos)

- Jumper wires (female-to-female)

## Section 1 — Raspberry Pi to PCA9685 (I²C)

The PCA9685 communicates over I²C. Connect four wires from the Raspberry Pi GPIO header to the PCA9685 input pins as shown below.

| Wire Color | Pi Physical Pin | Pi Function | PCA9685 Pin |
| --- | --- | --- | --- |
| Red | Pin 1 | 3.3V Power | VCC |
| Black | Pin 6 | Ground | GND |
| Yellow | Pin 3 | BCM 2 (SDA) | SDA |
| Green | Pin 5 | BCM 3 (SCL) | SCL |

Note: Use 3.3V (Pin 1) for VCC — not 5V. The PCA9685 logic runs at 3.3V. The V+ terminal on the board is for servo power only (see Section 2).

### I²C Pin Reference (Pi GPIO Header)

| Function | BCM Number | Physical Pin |
| --- | --- | --- |
| SDA (Data) | BCM 2 | Pin 3 |
| SCL (Clock) | BCM 3 | Pin 5 |
| Ground | — | Pin 6 (or any GND pin) |
| 3.3V Power | — | Pin 1 (or Pin 17) |

## Section 2 — Servo Motor Power Supply

Servo motors draw significant current and must be powered separately — do not power them from the Pi's 5V pins. Use a dedicated 5V–6V regulated supply connected to the PCA9685 V+ terminal block.

| Wire | From | To | Notes |
| --- | --- | --- | --- |
| Power (+) | PSU positive terminal | PCA9685 V+ | 5V–6V regulated |
| Ground (−) | PSU negative terminal | PCA9685 GND | Must share ground with Pi |

Note: Always connect the power supply ground (GND) to the same ground rail as the Raspberry Pi. Without a common ground, signals will not reference correctly and servos will behave erratically.

## Section 3 — Enable I²C on Raspberry Pi

I²C is disabled by default. Enable it before running any code.

Option A — raspi-config (recommended):

sudo raspi-config

Navigate to: Interface Options → I2C → Yes → Finish → Reboot

Option B — manual config.txt edit:

echo 'dtparam=i2c_arm=on' | sudo tee -a /boot/config.txt && sudo reboot

Verify the connection:

sudo i2cdetect -y 1

You should see address 0x40 (or 0x41–0x7F if address pins A0–A5 are bridged). If no address appears, recheck wiring.

## Quick Checklist

| # | Check | Status |
| --- | --- | --- |
| 1 | SDA → Pin 3 (BCM 2) | ☐ |
| 2 | SCL → Pin 5 (BCM 3) | ☐ |
| 3 | VCC → Pin 1 (3.3V) | ☐ |
| 4 | GND → Pin 6 | ☐ |
| 5 | Servo V+ connected to 5V–6V PSU | ☐ |
| 6 | PSU GND shares ground with Pi | ☐ |
| 7 | I²C enabled via raspi-config | ☐ |
| 8 | i2cdetect shows address 0x40 | ☐ |

Humanoid Robot Project  ·  github.com/ayushshah-xo/Humanoid-Robot-